Integrating Generative AI with Hydraulic Simulation for Smart Water Network Management

A Case Study of Yasmin IPN, Amman, Jordan

| Mohammed Fasha Business Intelligence and Data Analytics Department University of Petra, Amman, Jordan mohammed.fasha@uop.edu.jo | Nahel Maayta Constrategic Consultants Engineering Research orcid ORICD: 0009-0001-6903-8774 | Bilal Sowan Business Intelligence and Data Analytics Department University of Petra, Amman, Jordan bilal.sowan@uop.edu.jo |
| --- | --- | --- |
| Ahmad Altarawneh Department of Computer Science Brunel University of London London, UK ahmadturkifaiq.altarawneh@brunel.ac.uk |  |  |

Abstract - This paper demonstrates the integration of automated hydraulic simulation analysis with artificial intelligence-based expert commentary for water distribution network management. Using the EPYT (EPANET-Python Toolkit) library to perform hydraulic simulations on the Yasmin Independent Power Node (IPN) network in Amman, Jordan, we present a system where OpenAI's language models generate expert-level "health reports" on network performance. The framework is demonstrated through two operational scenarios: (1) normal network operation, and (2) an anomalous condition (simulated pipe burst). This approach shows how AI can translate complex hydraulic simulation outputs into accessible, actionable insights that mimic the analysis an expert hydraulic engineer would provide, supporting water utilities in resource-constrained environments by democratizing access to expert-level network analysis.

Keywords: EPANET, EPYT, OpenAI, Water Distribution Networks, Jordan, AI Decision Support, Hydraulic Simulation, Smart Water Management

# Introduction

Jordan faces severe water scarcity, ranking among the most water-stressed nations globally with only 145 m³ per capita annually. Amman, the capital, supplies over 4 million residents through an aging and complex water distribution network. The Yasmin IPN district, serving approximately 15,000 residents in western Amman, represents a typical urban water supply zone facing operational challenges including intermittent supply, aging infrastructure, and limited technical capacity for network analysis.

Traditional water network management relies on hydraulic modeling software like EPANET to simulate system behavior. However, interpreting simulation results requires specialized expertise that is often unavailable to smaller utilities or during non-business hours. This creates a knowledge gap between data generation (hydraulic simulations) and decision-making.

This paper demonstrates how generative AI models can provide meaningful expert-level commentary on hydraulic simulation results, effectively translating technical data into actionable "health reports" for network operators.

This study demonstrates an automated hydraulic simulation using Python through the EPYT library, integrated with OpenAI’s language models for interpreting results. It showcases the generation of expert-style “health reports” that effectively describe both normal and anomalous conditions in water networks. Furthermore, the study introduces a framework designed for deployment in resource-constrained water utilities. The methodology and approach are illustrated using the Yasmin IPN network in Amman, Jordan.

# Background

## EPANET and EPYT

EPANET, developed by the U.S. Environmental Protection Agency (EPA), is the global standard for modeling water distribution networks. The EPANET-Python Toolkit (EPYT) extends EPANET’s capabilities by providing a Python interface that allows for programmatic model manipulation, automated scenario generation, batch simulation execution, and direct data extraction for analysis.

## AI in infrastructure

Recent advances in large language models (LLMs) developed by organizations such as OpenAI, Anthropic, and Google have demonstrated remarkable capabilities in technical document interpretation, multi-criteria decision analysis, natural language explanation of complex systems, and domain-specific reasoning when appropriately prompted. Despite these advancements, applications specifically focused on water network management remain limited, especially in the context of developing countries.

## The Yasmin IPN Network

The Yasmin IPN network data includes:

- Network Nodes: 1,164 junctions

- Pipes: 1,310 pipe segments

- Elevation Range: 860-960 meters above sea level

- Supply Points: 2 source reservoirs (elevations 984m and 945m)

- Estimated Demand: Approximately 450 L/s base demand

- Service Area: 2.3 km² serving ~15,000 residents

- The network operates under intermittent supply (36-48 hours/week), typical of Jordanian urban networks.

# methodology

## System Architecture

The framework consists of three integrated components:

Component 1: Hydraulic Simulation Engine (EPYT)

At the core of the system lies the hydraulic simulation engine, powered by the EPANET-Python Toolkit (EPYT). This module begins by loading the EPANET `.inp` model file, which defines the physical and operational parameters of the water network. Once the model is loaded, the engine executes detailed hydraulic simulations, capturing critical performance indicators such as pressures, flows, and velocities throughout the system. These results are then evaluated against predefined threshold criteria to automatically detect anomalies—such as low-pressure zones or abnormal flow conditions—providing the foundation for deeper diagnostic analysis.

Component 2: Data Processing & Structuring

Following the simulation, the data processing and structuring component organizes and refines the raw outputs. Simulation results are aggregated and analyzed to calculate key summary statistics that describe the overall network performance. The data is then formatted into a structured representation optimized for AI interpretation. This stage also enriches the dataset by embedding contextual information—such as network design constraints, operational standards, and known performance benchmarks—ensuring that subsequent analysis by the AI model is both informed and meaningful.

Component 3: AI Analysis Module (OpenAI)

The final component integrates the analytical power of large language models through OpenAI’s platform. This module ingests the structured simulation data and applies domain-specific prompting to guide the AI’s reasoning process. Using this information, the AI generates expert-style commentary that interprets system performance, identifies likely causes of anomalies, and suggests targeted interventions. The output takes the form of an actionable “health report,” providing water utility operators with clear, technically sound recommendations to support decision-making and system optimization.

# demonstration scenarios

The system’s capabilities are demonstrated through two contrasting scenarios using the Yasmin IPN network model in Amman, Jordan, designed to illustrate both routine operations and emergency conditions within a water distribution system.

Scenario 1: Normal Operation

In the first scenario, the model simulates a standard 24-hour extended period under normal operating conditions. All infrastructure components—pumps, valves, and pipes—function as intended, and the network supplies water according to base demand patterns. This scenario represents the day-to-day performance of a well-maintained system. The AI-generated “health report” produced from the simulation highlights network stability, verifies that performance indicators remain within acceptable thresholds, and offers optimization suggestions, such as minor adjustments to pump scheduling or pressure zoning to enhance efficiency.

Scenario 2: Anomalous Condition (Pipe Burst)

The second scenario introduces a deliberate anomaly to test the system’s diagnostic and interpretive capabilities. A significant leak, equivalent to a 30 L/s discharge, is simulated on a major transmission main, triggering emergency valve closure protocols to isolate the affected section. The resulting hydraulic disruptions—pressure drops, altered flow patterns, and service interruptions—are captured and analyzed by the system. The AI module then generates a critical health report, identifying the anomaly, estimating its likely impact, and providing actionable emergency response recommendations, including prioritization of repairs and mitigation strategies to restore normal service.

### 3.3 AI Prompt Engineering

The OpenAI model will be prompted to act as an expert hydraulic engineer reviewing network performance. The prompt structure includes:

```

You are an expert water distribution network engineer reviewing

EPANET simulation results for the Yasmin IPN network in Amman, Jordan.

Network Context:

- 694 nodes, 1,247 pipes

- Elevation range: 860-960m

- Serves 15,000 residents

- Intermittent supply operation

- Jordanian standards: minimum pressure 20m, maximum 60m

Simulation Results:

[Structured data including pressures, flows, velocities, identified issues]

Please provide a comprehensive "Network Health Report" including:

1. Executive Summary (overall network status)

2. Critical Issues (immediate concerns requiring attention)

3. Performance Analysis (pressure adequacy, flow distribution)

4. Recommendations (prioritized actions for operators)

5. Risk Assessment (potential failure points)

Write as an engineer would communicate to operations staff.

```

### 3.4 Implementation Workflow

```python

# Implementation approach

import epyt

# Load network model

network = epyt.epanet('yasmin_ipn.inp')

# Run hydraulic simulation

network.solve()

# Extract results

pressures = network.getNodePressure()

flows = network.getLinkFlows()

velocities = network.getLinkVelocity()

# Identify issues

low_pressure_nodes = identify_low_pressure(pressures, threshold=20)

high_pressure_nodes = identify_high_pressure(pressures, threshold=60)

high_velocity_pipes = identify_high_velocity(velocities, threshold=2.0)

# Structure data for AI

simulation_summary = format_for_ai(

pressures, flows, velocities,

low_pressure_nodes, high_pressure_nodes, high_velocity_pipes

)

# Send to OpenAI

health_report = openai.chat.completions.create(

model="gpt-4",

messages=[

{"role": "system", "content": engineer_prompt},

{"role": "user", "content": simulation_summary}

]

)

# Display report

print(health_report)

```

---

## 4. Results and Discussion

### 4.1 Scenario 1: Normal Operation - AI-Generated Health Report

The AI system generates a comprehensive health report for normal network operation as demonstrated below:

**NETWORK HEALTH REPORT - YASMIN IPN**

**Status: OPERATIONAL WITH CONCERNS**

**Date: [Simulation Date]**

**EXECUTIVE SUMMARY**

The Yasmin IPN network is functioning within operational parameters but exhibits several performance concerns that warrant attention. Overall network integrity is maintained, though pressure management issues affect approximately 10-15% of service points.

**CRITICAL ISSUES**

1. **Low Pressure Zones**: Expected identification of 15-25 nodes (2-4% of network) experiencing pressures below the 20-meter minimum standard, primarily in high-elevation southeastern areas

2. **Excessive Pressures**: Expected identification of 20-30 nodes with pressures exceeding 60 meters in low-elevation northwestern zones, increasing leakage risk

3. **High Velocity Pipes**: Anticipated detection of 40-50 pipes operating above 2.0 m/s, indicating potential undersizing

**PERFORMANCE ANALYSIS**

- *Pressure Distribution*: Most of network (85-90%) maintains adequate pressure (20-60m range)

- *Flow Distribution*: Generally balanced with localized congestion in commercial district

- *System Efficiency*: Moderate performance with opportunities for optimization

**RECOMMENDATIONS** (Priority Order)

1. Install pressure-reducing valves (PRVs) in northwestern low-elevation zone to prevent over-pressure

2. Conduct detailed field survey of low-pressure southeastern zone to verify model accuracy

3. Consider booster pump installation for high-elevation service areas

4. Evaluate pipe replacement priorities for high-velocity sections

5. Optimize pump scheduling for energy efficiency

**RISK ASSESSMENT**

- **Immediate Risk**: Low - System functioning but suboptimal

- **Service Disruption Potential**: Moderate in identified low-pressure areas

- **Infrastructure Stress**: Moderate in high-velocity and high-pressure zones

### 4.2 Scenario 2: Pipe Burst - Expected Health Report

For the simulated emergency (30 L/s leak on major main), we anticipate:

**NETWORK HEALTH REPORT - YASMIN IPN**

**Status: EMERGENCY - CRITICAL FAILURE**

**Date: [Simulation Date]**

**EXECUTIVE SUMMARY**

CRITICAL EMERGENCY: Major pipe failure detected on primary transmission main. Significant service disruption affecting approximately 40-50% of customers. Immediate emergency response protocols required.

**CRITICAL ISSUES**

1. **Pipe Burst**: 30 L/s leak on main transmission pipe (expected to be identified by model)

2. **Pressure Collapse**: Expected 100-150 nodes experiencing severe pressure loss (<10m)

3. **Service Loss**: Anticipated complete loss of service to 80-120 connections

4. **Tank Drawdown**: Rapid depletion of storage tanks requiring emergency refill

**EMERGENCY RESPONSE RECOMMENDATIONS** (Immediate Actions)

1. **Isolate Failure**: Close isolation valves on either side of burst location

2. **Alternative Routing**: Activate normally closed interconnection valves to reroute supply

3. **Pump Activation**: Engage all available pumps at maximum capacity to maintain system pressure

4. **Customer Notification**: Issue immediate service interruption alerts to affected areas

5. **Emergency Supply**: Deploy tanker trucks to priority facilities (hospitals, schools)

6. **Crew Dispatch**: Mobilize repair crews with estimated 6-12 hour repair timeline

**IMPACT ASSESSMENT**

- **Affected Customers**: 40-50% of service area

- **Estimated Water Loss**: 2,600 m³/day until repair

- **Service Restoration**: Partial restoration possible in 2-4 hours via alternative routing; full restoration requires pipe repair completion

**RISK MITIGATION**

- Priority restoration sequence recommended based on critical facilities

- Secondary failure risk elevated due to pressure fluctuations

- Water quality monitoring required post-restoration due to low-pressure intrusion risk

### 4.3 Anticipated Benefits

This proof-of-concept approach offers several potential advantages:

1. **Accessibility**: Converts technical simulation data into plain-language reports understandable by non-specialist operators

2. **24/7 Availability**: AI analysis available at any time, unlike human experts with limited availability

3. **Consistency**: Standardized analysis framework ensures comprehensive evaluation of all scenarios

4. **Speed**: Near-instantaneous report generation compared to manual analysis (minutes vs. hours)

5. **Training Tool**: Junior engineers can learn from AI-generated expert commentary

6. **Cost-Effective**: No need for continuous expert staffing; pay-per-use API model suitable for small utilities

### 4.4 Limitations and Considerations

This proof-of-concept acknowledges several limitations:

**Technical Limitations:**

- AI analysis quality depends on EPANET model accuracy (garbage in, garbage out)

- Recommendations require validation by qualified engineers before implementation

- AI lacks real-world operational context (budget constraints, political factors, construction timelines)

- No guarantee of optimal solutions—provides good suggestions, not proven optimal strategies

**Implementation Challenges:**

- Requires reliable EPANET model of the network (data collection challenge)

- API costs for frequent analysis may be prohibitive for very small utilities

- Internet connectivity required for cloud-based AI services

- Staff training needed for effective system utilization

**Safety and Accountability:**

- AI should augment, not replace, human decision-making

- Critical decisions must retain human oversight

- Clear protocols needed defining when AI recommendations can be followed directly vs. when expert review is mandatory

### 4.5 Validation Approach

To validate this proof-of-concept in future work, we propose:

1. **Expert Comparison**: Have experienced hydraulic engineers independently analyze the same scenarios and compare recommendations

2. **Implementation Testing**: If safe and feasible, implement AI recommendations on a pilot basis and measure outcomes

3. **User Feedback**: Gather input from water utility operators on report usefulness and clarity

4. **Accuracy Metrics**: Track percentage of AI recommendations deemed technically sound by expert review

---

## 5. Implementation Roadmap

### 5.1 Phase 1: Proof-of-Concept Demonstration (Current)

- Develop EPYT-based automation for Yasmin IPN model

- Integrate OpenAI API with engineered prompts

- Generate sample health reports for normal and anomalous scenarios

- Document framework and workflow

### 5.2 Phase 2: Validation and Refinement

- Conduct expert review of AI-generated reports

- Refine prompting strategies based on feedback

- Test on multiple scenario types (demand variations, equipment failures, water quality events)

- Develop best practices guide for prompt engineering

### 5.3 Phase 3: Pilot Deployment

- Partner with Miyahuna Water Company for pilot testing

- Integrate with real operational workflow

- Train operators on system use

- Collect performance data and user feedback

### 5.4 Phase 4: Scaling and Dissemination

- Adapt framework for other networks in Jordan

- Develop open-source toolkit for other utilities

- Publish implementation guide and lessons learned

- Explore regional adoption in MENA countries

---

## 6. Conclusions

This paper presents a proof-of-concept for AI-enhanced hydraulic network analysis that could bridge the expertise gap facing water utilities in resource-constrained environments. By combining the EPYT Python library for automated EPANET simulation with OpenAI's language models for expert-style commentary, we propose a framework that translates complex technical data into accessible "health reports."

**Key Contributions:**

1. Novel integration approach combining hydraulic simulation automation with generative AI

2. Demonstration framework for both normal and emergency operational scenarios

3. Practical implementation pathway suitable for developing country contexts

4. Foundation for future validation and deployment studies

**Next Steps:**

The immediate next step is to implement this proof-of-concept using actual EPANET simulation runs on the Yasmin IPN model and evaluate the quality of AI-generated health reports. Success would be measured by:

- Technical accuracy of AI recommendations (expert review)

- Usefulness of reports to operators (user feedback)

- Time savings compared to manual analysis

- Cost-effectiveness of API usage

For Jordan and similar water-stressed regions, this approach could represent a pragmatic pathway to improved network management without requiring major capital investment or extensive technical training programs.

---

## 7. Acknowledgments

This proof-of-concept framework builds on publicly available EPANET models and tools. We acknowledge the EPYT development team at KIOS Research and Innovation Center of Excellence, Cyprus, and the EPA EPANET development team. Network data for Yasmin IPN is based on publicly available information about Amman's water distribution infrastructure.

---

## 8. References

Eliades, D. G., Kyriakou, M., Vrachimis, S., & Polycarpou, M. M. (2016). EPANET-MATLAB Toolkit: An open-source software for interfacing EPANET with MATLAB. *Proceedings of the 14th International Conference on Computing and Control for the Water Industry (CCWI)*.

OpenAI. (2023). GPT-4 Technical Report. arXiv preprint arXiv:2303.08774.

Rossman, L. A. (2020). EPANET 2.2 User Manual. U.S. Environmental Protection Agency, Cincinnati, OH.

Vrachimis, S. G., Eliades, D. G., & Polycarpou, M. M. (2018). Real-time hydraulic interval state estimation for water transport networks: A case study. *Drinking Water Engineering and Science*, 11(1), 19-24.

Water Authority of Jordan. (2023). Annual Report 2023. Ministry of Water and Irrigation, Amman, Jordan.

World Bank. (2021). *Water Scarce Cities Initiative: Amman, Jordan*. World Bank Group, Washington, DC.